GS/OS AppleDisk5.25 Driver ERS

Ver. 2.05

(For System Disk 6.0)

© Copyright 1987 - 1990

Apple Computer, Inc.

All Rights Reserved

AppleDisk 5.25 Driver

Table of Contents

About This Document	1

General Information	1

Disk][ Limitations	1

AppleDisk5.25 Driver FileType and AuxType	1

Device Driver Structure	1

About the Driver Header	2

About the DIB	3

Disk][ Media	5

How the AppleDisk5.25 Driver is called	7

AppleDisk5.25 Driver Calls	7

Driver_Open	8

Driver_Read	9

Driver_Write	10

Driver_Close	11

Driver_Status	12

Device Status	13

Return Configuration Parameters	13

Wait / No Wait Status	14

Get Format Options	14

Get Last Volnum	15

Driver_Control	16

Reset Device	17

Format Device	17

Eject	17

Set Configuration Parameters	17

Wait / No Wait Mode	17

Set Format Options	17

Assign Partition Owner	17

Arm Signal	18

Disarm Signal	18

Set Next Volnum	18

Driver_Flush	19

Device Driver Error Codes	20

.c.About This Document

This document is intended to describe the GS/OS AppleDisk5.25 driver calls.  Included is a brief description of how each call is implemented with respect to the AppleDisk5.25.  Also included is a description of the physical and logical formats used with the AppleDisk5.25 media.

.c.General Information

The AppleDisk5.25 driver is a loaded driver which does not require a supervisory driver to conduct I/O transactions with the AppleDisk5.25 device.  It supports up to fourteen Disk][ devices and should operate equally well with a AppleDisk5.25 Interface Card or IWM interface.  The AppleDisk5.25 driver operates independent of the system speed and does not have the resident slot limitation inherent in the Apple//GS.  (The Apple//GS normally only allows Disk][ devices in slots 4 through 7 in fast mode.  This driver operates with Disk][ devices in slots 1 through 7 in fast mode with either one or two Disk][ devices per slot.)

.c.Disk][ Limitations

The Disk][ device provides no means for detection of disk switched errors.  A simulation of disk switched is provided that will force any file system translator interfacing to the Disk][ to identify the volume currently online.  Simulation of disk switched errors is adequate to force volume identification but is not adequate to validate the integrity of the cache.  For this reason, the AppleDisk5.25 driver does not implement caching.  Additionally, a status call will never return a disk switched status.

.c.AppleDisk5.25 Driver FileType and AuxType

The AppleDisk5.25 driver is compacted and has a filetype of $BB as do all GS drivers.  The AuxType for the AppleDisk5.25 driver has been set to $010E indicating that the driver is a GS/OS driver supporting a maximum of 14 ($0E) devices.

.c.Device Driver Structure

The AppleDisk5.25 driver consists of a driver header, configuration parameter list, device information block and the driver code segment.  No scripts exist for the AppleDisk5.25 driver, therefore, no script segment has been provided.  No configuration parameters exist for the AppleDisk5.25 driver.  The configuration parameter list for each Disk][ device has a length word of NIL.

.c.About the Driver Header

The header is used when loading the driver.  It indicates where the configuration parameter lists and DIBs are located.  The device dispatcher loads only the driver, DIBs and configuration parameter lists using an initial_load call to the system loader.  The header contains the following information:

Word	Offset to 1st DIB

Word	Count of number of devices = 14

Word	Offset to 1st configuration parameter list for device #1

Word	Offset to 1st configuration parameter list for device #2

Word	Offset to 1st configuration parameter list for device #3

Word	Offset to 1st configuration parameter list for device #4

Word	Offset to 1st configuration parameter list for device #5

Word	Offset to 1st configuration parameter list for device #6

Word	Offset to 1st configuration parameter list for device #7

Word	Offset to 1st configuration parameter list for device #8

Word	Offset to 1st configuration parameter list for device #9

Word	Offset to 1st configuration parameter list for device #10

Word	Offset to 1st configuration parameter list for device #11

Word	Offset to 1st configuration parameter list for device #12

Word	Offset to 1st configuration parameter list for device #12

Word	Offset to 1st configuration parameter list for device #14

.c.About the DIB

The DIB must contain the following information:

The link pointer is a longword pointing to the next DIB for device drivers supporting multiple DIBs.  If the device driver supports only a single DIB then the link pointer should be set to NIL.  This is used to install the device drivers into the device list.  This pointer does not imply any links between devices.

The entry pointer is a longword pointer to the device driver's entry point.  The AppleDisk5.25 driver has a common entry point referenced by the DIB for each Disk][ device.

The device characteristics word parameter describes features that may or may not be supported by the device.  The AppleDisk5.25 driver's device characteristics are set to $03EC indicating that the Disk][ is not speed dependent, is a block device and supports removable media, formatting, read and write operations.  A pictorial representation of the device characteristics word is shown below:

Definition of the Speed Group bits are as follows:

00	1 Mhz Device

01	2.6 Mhz Device

10	>2.6 Mhz Device

11	Device is not speed dependent

Speed independence is achieved through use of the system service call "set_speed" prior to executing time critical sections of the code.

The Block Count is a longword parameter which is only used with block devices.  It indicates the total number of blocks accessible on the device.  The AppleDisk5.25 driver indicates the number of 512 byte blocks in the DIB as 280 blocks.

Device Name is a 32 byte field which contains a count byte followed by a device name encoded in up to 31 bytes of ASCII.  Note that the initial '.' is not included in the device name.  The device name must be in upper case with the MSB off.  The AppleDisk5.25 driver returns a seven character name "AppleDisk5.25x" where "x" is a unique character for each Disk][ device.

Slot Number is the number of the slot where the device hardware resides.  Bits 0 through 2 indicate the slot while bit 3 indicates that the slot is internal or external.

Unit Number is the device number within the slot.  This is not a global unit number relating to the device list.

Device Version Number is a word parameter which indicates the version number of either a loaded or generated driver.  Generated drivers may use the version number obtained from the slot resident firmware interface.  The most significant nibble of the version indicates the major release version while the next two most significant nibbles indicate the minor release version.  The least significant nibble indicates: E=Experimental, A=Alpha, B=Beta, 0=Final.  The first release of the AppleDisk5.25 driver has a version of $001A indicating a first alpha phase release.

Device ID Number is a word specifying the type of device.  The AppleDisk5.25 driver has a device id of $0000.

The AppleDisk5.25 driver does not support linked devices.  Both the Head and Forward links in the DIB are set to a NIL value.

Two additional words have been reserved in the DIB for future expansion.  These words are set to NIL.

.c.Disk][ Media

The AppleDisk5.25 driver supports only 35 track 16 sector media.  Media is formatted with a physical 1:1 interleave.  Logical interleave is achieved by using one of two interleave translation tables.   Dos operates on 256 byte sectors.  ProDOS and Pascal operate on 512 byte blocks consisting of two contiguous logical sectors.  Both ProDOS and Pascal use a common logical sector interleave of 2:1 while Dos uses a logical sector interleave of 14:1.  Interleave translation table selection is based on the block size set as input to a media access call.  Logical to physical sector translations are shown in the figures below:

Each sector consists of a self synchronization gap, address field, second self synchronization gap, data marks and data field as shown in the figure below.

.c.How the AppleDisk5.25 Driver is called

Applications may access the Disk][ device either through a file system translator (such as ProDOS, Pascal or DOS3.3) or via the Device Manager calls (D_INFO, D_READ or D_WRITE).

.c.AppleDisk5.25 Driver Calls

All drivers will accept a standard set of calls.  AppleDisk5.25 driver calls accessible to an FST or the Device Manager will include :

• DRIVER_OPEN	(FST only)

• DRIVER_READ

• DRIVER_WRITE

• DRIVER_CLOSE	(FST only)

• DRIVER_STATUS

• DRIVER_CONTROL

• DRIVER_FLUSH 	(FST only)

The details of each driver call will be described individually.  Each of these calls will be described in detail on the following pages.

.c2.Driver_Open

Call Parameters :	Device Number	≠ $0000

Call Number	= $0001

DIB Pointer

Device Number:	This word parameter specifies which device is to be accessed by the call.  This parameter must be a non-zero value.

Call Number:	This word parameter specifies which type of call is to be issued to the device.

DIB Pointer:	This longword points to the device information block for the device being accessed.

This call has no function with block devices.  The AppleDisk5.25 driver will return with no error.

.c2.Driver_Read

Call Parameters :	Device Number	≠ $0000

Call Number	= $0002

Buffer Pointer

Request Count

Transfer Count

Block Size	≠ $0000

FST Number

DIB Pointer

Device Number:	This word parameter specifies which device is to be accessed by the call.  This parameter must be a non-zero value.

Call Number:	This word parameter specifies which type of call is to be issued to the device.

Buffer Pointer:	This is a longword pointer to memory where the data is to be written to after being read from the device.

Request Count:	This is a longword specifying the number of bytes that the driver is being requested to transfer from the device to the buffer specified by buffer pointer.

Transfer Count:	This is a longword returned by the call that indicates the number of bytes actually transferred.

Block Number:	This longword parameter specifies the logical address within the block device from which data is to be transferred from.  This parameter has no application in character device drivers.

Block Size:	This word parameter specifies the size of the block addressed by the block number.  This parameter must be a non-zero value for block devices.  This parameter must be set to a value of zero for character devices.

FST Number:	This word parameter specifies the File System Translator that owns the volume for which the block is being transferred.

DIB Pointer:	This longword points to the device information block for the device being accessed.

This call returns the requested number of bytes from the disk starting at the block number specified.  The request count must be an integral multiple of the block size.  If during a multiple block transaction the block address exceeds the block address range for the Disk][ then a bad block error will be returned with the transfer count indicating the number of bytes read successfully from the Disk][ device.  Note that the AppleDisk5.25 driver supports a block size of 256 bytes or 512 bytes and block counts of 560 and 280 blocks respectively.  Logical interleaving on the disk varies with the block size.  It should also be noted that if the Disk][ has not had a media access call in the one second previous to issuing this call then a disk switched error will be returned except if the call is issued through the device manager.

.c2.Driver_Write

Call Parameters :	Device Number	≠ $0000

Call Number	= $0003

Buffer Pointer

Request Count

Transfer Count

Block Size	≠ $0000

FST Number

DIB Pointer

Device Number:	This word parameter specifies which device is to be accessed by the call.  This parameter must be a non-zero value.

Call Number:	This word parameter specifies which type of call is to be issued to the device.

Buffer Pointer:	This is a longword pointer to memory where the data is to be written to after being read from the device.

Request Count:	This is a longword specifying the number of bytes that the driver is being requested to transfer from the device to the buffer specified by buffer pointer.

Transfer Count:	This is a longword returned by the call that indicates the number of bytes actually transferred.

Block Number:	This longword parameter specifies the logical address within the block device from which data is to be transferred from.  This parameter has no application in character device drivers.

Block Size:	This word parameter specifies the size of the block addressed by the block number.  This parameter must be a non-zero value for block devices.  This parameter must be set to a value of zero for character devices.

FST Number:	This word parameter specifies the File System Translator that owns the volume for which the block is being transferred.

DIB Pointer:	This longword points to the device information block for the device being accessed.

This call write the requested number of bytes to the disk starting at the block number specified.  The request count must be an integral multiple of the block size.  If during a multiple block transaction the block address exceeds the block address range for the Disk][ then a bad block error will be returned with the transfer count indicating the number of bytes read successfully from the Disk][ device.  Note that the AppleDisk5.25 driver supports a block size of 256 bytes or 512 bytes and block counts of 560 and 280 blocks respectively.  Logical interleaving on the disk varies with the block size.  It should also be noted that if the Disk][ has not had a media access call in the one second previous to issuing this call then a disk switched error will be returned except if the call is issued through the device manager.

.c2.Driver_Close

Call Parameters :	Device Number	≠ $0000

Call Number	= $0004

DIB Pointer

Device Number:	This word parameter specifies which device is to be accessed by the call.  This parameter must be a non-zero value.

Call Number:	This word parameter specifies which type of call is to be issued to the device.

DIB Pointer:	This longword points to the device information block for the device being accessed.

This call has no function with block devices.  The AppleDisk5.25 driver will return with no error.

.c2.Driver_Status

Call Parameters :	Device Number	≠ $0000

Call Number	= $0005

Status List Pointer

Request Count

Transfer Count

Status Code

DIB Pointer

Device Number:	This word parameter specifies which device is to be accessed by the call.  This parameter must be a non-zero value.

Call Number:	This word parameter specifies which type of call is to be issued to the device.

Status List Pointer:	This is a longword pointer to memory where the status list is to be written into.

Request Count:	This longword parameter passed to the call indicates the number of bytes to be transferred.  If the request count is smaller than the minimum buffer size required by the call, an error will be returned.

Transfer Count:	This is a longword returned by the call that indicates the number of bytes actually transferred.

Status Code:	This is a word parameter specifying the type of status request.  Status codes of $0000 through $7FFF are standard status calls that must be supported by device drivers.  Devices supporting device specific status calls should use status codes in the range of $8000 through $FFFF.  A list of standard status calls is shown below:

$0000	Device Status

$0001	Return Configuration Parameters

$0002	Return Wait / No Wait Status

$0003	Get Format Options

$0004 - $7FFF	Reserved - these status codes to be assigned

by Apple Computer, Inc.

$8000 - $FFFF	Device Specific

DIB Pointer:	This longword points to the device information block for the device being accessed.

This call is used to obtain current status information from the device or the driver. Extensions to the standard set of calls which transfer data or status information from the device or device driver are supported as a subset of this call.  The device driver is responsible for validating the status code prior to executing the requested status call.  If an invalid status code is passed to the driver, the driver should return a 'BAD CODE' error.  The device dispatcher will set the transfer count to zero prior to calling the device driver.  The device driver should set the transfer count to the number of bytes returned as a result of the status call.

.c3.Device Status

This call returns a general status followed by a longword specifying the number of blocks supported by the device.  Write protect reflects the state of the write protect sense line on the previous media access.

Bit 15	0 = Block count certain, 1 = Block count uncertain for block size

Bit 14	0

Bit 5-13	Reserved (currently read as zero)

Bit 4	1 = Disk in drive, 0 = Disk not in drive

Bit 3	Reserved (currently read as zero)

Bit 2	1 = Write protected, 0 = Write enabled

Bit 1	1 = Device currently interrupting

Bit 0	1 = Disk has been switched

Note that there is no way to validate media insertion on a Disk][.  Bit 4 of the device status word will always be set to a '1'.

Status List:	Word	General status word

Long	Number of blocks supported by device

.c3.Return Configuration Parameters

This call returns a byte count as the first word in the status list which indicates the length of the configuration parameter list in bytes.  The configuration parameters will be placed into the status list contiguous to the byte count.  The Disk][ has no parameters in it's configuration parameter list and will return with a length word of zero and transfer count of $00000002.

Status List:	Word	Length of configuration parameter list

Data	Data returned from configuration parameter list

.c3.Wait / No Wait Status

Block devices only operate in WAIT mode.  This call always returns a word of $0000 and a transfer count of $00000002 for Disk][ devices.

Status List:	Word	Wait status

.c3.Get Format Options

This call returns a list of formatting options that may be selected using a Set_Format_Options call prior to issuing a format call to a block device.  These parameters may include such variables as format environment, number of blocks, block size, and interleave.  Devices that do not support media variables will return with a transfer count of zero and no error.  The format of the status list on return from this call when a device does support media variables is as follows:

Returned List:	Word	Number of entries in list

Word	Number of displayed entries in list

Word	Recommended Default Option

Word	Option that current online media is formatted with

Then each entry in the list consists of 16 bytes containing the following 5 fields:

Word	Media variables reference number

Word	Reference number of linked entry

Word	Flags

Long	Number of blocks supported by device

Word	Block Size

Word	Interleave Factor

Word	Media size (block count * block size)

Flags word definition is as follows:	Bits 0 - 1	Format Type

Bits 2 - 3	Size Multiplier

Bits 4 - 15	Reserved (must be zero)

Format Type definition is as follows:	00	Universal Format

01	Apple Format

10	NonApple Format

11	Not valid

Size Multiplier definition is as follows:	00	Size in bytes

01	Size in kilobytes

10	Size in megabytes

11	Size in gigabytes

The AppleDisk5.25 driver returns format options as follows:

Transfer count = $00000028 (40 bytes returned in list)

Returned List:	$0002	Two entries in list

$0001	Only one display entries

$0001	Recommended default is option #1

$0000	Current media formatted is unknown

$0001	Refnum = Option #1

$0002	LinkRef = Option #2

$0004	Universal format / size in kilobytes

$00000118	Block count = 280

$0200	Block size = 512 bytes

$0000	Interleave factor = n/a (fixed physical interleave)

$008C	Media size = 140 kilobytes

$0002	Refnum = Option #2

$0000	LinkRef = NIL

$0004	Universal format / size in kilobytes

$00000230	Block count = 560

$0100	Block size = 256 bytes

$0000	Interleave factor = n/a (fixed physical interleave)

$008C	Media size = 140 kilobytes

.c3.Get Last Volnum; ($8001)

This is the only device-specific status call that the AppleDisk5.25 driver supports.  It returns the volume number (read from the address field of a sector) of the last accessed disk in the status list.  The RequestCount field of the parameter block must be 2.  If no disk access has occurred prior to issuing this call, a volume number of 0 will be returned.

Status List:	Word	Volume number

.c2.Driver_Control

Call Parameters :	Device Number	≠ $0000

Call Number	= $0006

Control List Pointer

Request Count

Transfer Count

Control Code

DIB Pointer

Device Number:	This word parameter specifies which device is to be accessed by the call.  This parameter must be a non-zero value.

Call Number:	This word parameter specifies which type of call is to be issued to the device.

Control List Pointer:	This is a longword pointer to memory where the control list is to be read from.

Request Count:	This longword parameter passed to the call indicates the number of bytes to be transferred.  If the request count is smaller than the minimum buffer size required by the call, an error will be returned.

Transfer Count:	This is a longword returned by the call that indicates the number of bytes actually transferred.

Control Code:	This is a word parameter specifying the type of control request.  Status codes of $0000 through $7FFF are standard control calls that must be supported by device drivers.  Devices supporting device specific control calls should use control codes in the range of $8000 through $FFFF.  A list of standard control calls is shown below:

$0000	Reset Device

$0001	Format Device

$0002	Eject

$0003	Set Configuration Parameters

$0004	Set Wait / No Wait Mode

$0005	Set Format Options

$0006	Assign Partition Owner

$0007	Arm Event

$0008	Disarm Event

$0009 - $7FFF	Reserved - these status codes to be assigned by 		Apple Computer, Inc.

$8000 - $FFFF	Device Specific

DIB Pointer:	This longword points to the device information block for the device being accessed.

This call is used to send control information to the device or the device driver.  Extensions to the standard set of calls which transfer data or control information to the device or device driver are supported through the use of device specific control codes.

The device driver is responsible for validating the control code and control list length prior to executing the requested control call.  If an invalid control code is passed to the driver, the driver should return a 'BAD CODE' error.  If an invalid control list length is passed to the driver, the driver should return a 'BAD PARAMETER' error.  The device driver should set the transfer count to the number of bytes processed as a result of a successful control call.

.c3.Reset Device

This control call is used to reset a particular device to it's default settings.  This call has no function with the AppleDisk5.25 driver and returns with no error.

Control List:	Word	Length of control list ($0000)

.c3.Format Device

This control call is used to format the media used by a block device.  This call is not linked to any particular file system.  It simply prepares all blocks on the media for reading and writing.

Control List:	Word	Length of control list ($0000)

.c3.Eject

The Disk][ device does not have any mechanism apart from the user's hand for ejecting media.  This call has no function with the AppleDisk5.25 driver and returns with no error.

Control List:	Word	Length of control list ($0000)

.c3.Set Configuration Parameters

This call has no function with the AppleDisk5.25 driver and returns with no error.

Control List:	Word	Length of configuration parameter list ($0000)

Data	Configuration Parameter List Data

.c3.Wait / No Wait Mode

All block devices including the Disk][ operate in WAIT mode only.  Setting the AppleDisk5.25 driver to wait mode results in no error.  If a call is issued to set the AppleDisk5.25 driver to no wait mode then a bad parameter error will be returned.

Control List:	Word	Length of control list ($0002)

Word	Wait / No Wait Mode

.c3.Set Format Options

This call has no function with the AppleDisk5.25 driver since only a single fixed physical interleave is supported.  This call returns with no error.

Control List:	Word	Length of control list ($0004)

Word	Format_RefNum

Word	Interleave_Factor

.c3.Assign Partition Owner

This call has no function with the AppleDisk5.25 driver and returns with no error.

Control List:	String	Class 1 string specifying partition owner

.c3.Arm Signal

This call has no function with the AppleDisk5.25 driver and returns with no error.

Control List:	Word	Signal Code

Word	Priority

Longword	Signal Handler Address

.c3.Disarm Signal

This call has no function with the AppleDisk5.25 driver and returns with no error.

Control List:	Word	Signal Code

.c3.Set Next Volnum; ($8001)

This device specific control call allows the caller to specify the volume number to use the next time the Format control call is made.  The volume number specified must be in the range 0-254.  If 0, it will default to 254.  The Format call will reset this to zero after performing the format.  Therefore, the volume number must be set explicitly each time a disk is to be formatted if the volume number is not to be defaulted.

Control List	Word	Volume number

.c2.Driver_Flush

Call Parameters :	Device Number	≠ $0000

Call Number	= $0007

DIB Pointer

Device Number:	This word parameter specifies which device is to be accessed by the call.  This parameter must be a non-zero value.

Call Number:	This word parameter specifies which type of call is to be issued to the device.

DIB Pointer:	This longword points to the device information block for the device being accessed.

This call has no function with the AppleDisk5.25 driver and returns with no error.

.c.Device Driver Error Codes

All error codes listed below must be supported by device drivers wherever applicable.  All block device drivers must support disk switched errors without exception.  Please take note that the error codes are returned from a device driver must have the high byte cleared.  The device dispatcher maintains certain error codes under certain conditions.  Device dispatcher error codes are passed in the upper byte of the accumulator.

Error Code	Description	Mnemonic

$0000	No error occurred	NO_ERROR

$0010	Device not found	DEV_NOT_FOUND

$0011	Invalid Device Number	INVALID_DEV_NUM

$0020	Invalid request	DRVR_BAD_REQ

$0021	Invalid control or status code	DRVR_ BAD_CODE

$0022	Invalid parameter	DRVR_ BAD_PARM

$0023	Device not open (character driver only)	DRVR_ NOT_OPEN

$0024	Device already open (character driver only)	DRVR_ PRIOR_OPEN

$0026 	Resource not available	DRVR_ NO_RESRC

$0027	I/O error	DRVR_ IO_ERROR

$0028	Device not connected	DRVR_ NO_DEV

$0029	Device is busy	DRVR_BUSY

$002B	Write Protected (block driver only)	DRVR_ WR_PROT

$002C	Invalid Byte Count	DRVR_ BAD_COUNT

$002D	Invalid Block Number (block driver only)	DRVR_ BAD_BLOCK

$002E	Disk Switched (block driver only)	DRVR_ DISK_SW

$002F	Device Off Line or No Media Present	DRVR_ OFF_LINE

$004E	Invalid access or access not allowed	INVALID_ACCESS

$0058	Not a block device	NOT_BLOCK_DEV

$0060	Data is unavailable	DATA_UNAVAIL