-----------------------------------

DATE:     April 13,1982

PRODUCT:  WordStar

CATEGORY: CORRECTION

Revision: All

PROBELM:  ETX/ACK & XON/XOFF protocols through CP/M LIST DEVICE

REF:      USER4, LISINP:, XON/XOFF, ETX/ACK, LST:, PRINTER

Description:

This bug only affects printers which are sent output characters through

the standard CP/M list (LST:) device and the users wishes to implement

the XON/OFF or ETX/ACK protocols.

The list device driver in USER 4 includes a patchable item called LISINP:

which is necessary to provide a routine to retreive the ACK character from

the printer.

Unfortunately this routine is never called by WordStar.  The fix is as

follows:

A>SID WSOVLY1.OVR<CR>

SID VERS 1.4

NEXT   PC   END

8600  0100  BCFF

#S739B<CR>

739B  FA CA<CR>

739C  25 .<CR>

#S7992<CR>

7992  FA CA<CR>

7993  25 .<CR>

#G0<CR>

A>SAVE 133 WSOVLY1.OVR<CR>

EAS

-----------------------------------

;DATE:     April 13,1982

;

;PRODUCT:  WordStar

;CATEGORY: MODIFICATION

;Revision: 3.0 only

;Subject:  SYSTEM FILE DISPLAY SUPPRESSION IN WS DIRECTORY

;

;  THIS PATCH WILL SUPPRESS DISPLAY OF ANY FILES ASSIGNED TO BE SYSTEM FILES

;  UNDER CP/M 2.XX (HAVEN'T TESTED MP/M) WITH THE COMMAND:

;

;		STAT FN.FT $SYS

;

;		PATCH IS FOR WORDSTAR 3.00	EAS  02/03/82

;

;    THE PATCH REPRESENTS 31 BYTES OF CODE IN THE USER PATCH AREA (USER1,

;    MORPAT:) ORGANIZED TO EXIST AT THE HIGHEST LOCATION AVAILABLE SO THAT

;    CURRENTLY EXISTING PATCHES WHICH DO NOT EXTEND PAST LOCATION 33CH WILL

;    NOT BE AFFECTED BY THE INSTALLATION OF THIS SYSTEM FILE SUPPRESSION.

;

;                             Enjoy,

;                                       Evan

;

DELENTX	EQU	2C28H

DELENTY	EQU	2C2BH

DTAB	EQU	2C69H

DENTPT	EQU	36D1H

DOSNX	EQU	2C8AH

NDTST	EQU	2BF6H

ORG	33DH

;

MORPAT: PUSH	D

INX	D

LDAX	D

ANI	80H

POP	D

JNZ	NODISP

LXI	H,DTAB

JMP	DELENTY

;

NODISP:	POP	H

POP 	B

XCHG

LHLD	DENTPT

XCHG

PUSH	H

CALL	DOSNX

POP	H

POP	D

JMP	NDTST

;

ORG	DELENTX

;

JMP	MORPAT

END

;DATE:     April 13,1982

;

;PRODUCT:  WordStar

;CATEGORY: MODIFICATION

;Revision: 3.0 only

;Subject:  Formmated output from printer driver

;

*************************************************

*	PRINT TO DISK WITH WORDSTAR 3.0		*

*************************************************

;

; THIS PROGRAM TAKES WHAT WORDSTAR SENDS TO THE LIST DEVICE

; TO A FILE CALLED FILEOUT.  THIS MEANS YOU CAN PRINT FILES

; DIRECTLY TO THE DISK AND THEN PRINT THEM LATER USING PIP

; OR AN EQUIVALENT.  ALSO GOOD FOR SEEING EXACTLY WHAT WORDSTAR

; DOES FOR ANY SPECIFIC PRINTER INSTALLATION.  INSTALL

; WORDSTAR FOR THE PRINTER OF YOUR CHOICE, NO PROTOCOL,

; AND CP/M "LST:" DEVICE DRIVER.  THIS PROGRAM USES

; PBGMEM, SO BE SURE TO SAVE ENOUGH PAGES TO HOLD THE

; PATCHES.

;

*************************

*	EQUATES		*

*************************

;

DEFBUF	EQU	0080H		; DEFAULT BUFFER USED FOR FILE I/O

BDOS	EQU	5		; LOCATION OF BDOS JUMP VECTOR

CLOSEF	EQU	16		; CLOSE FILE FUNCTION

DELETEF	EQU	19		; DELETE FILE FUNCTION

MAKEF	EQU	22		; MAKE FILE FUNCTION

WRITEF	EQU	21		; WRITE SEQUENTIAL

SETDMA	EQU	26		; SET DMA ADDRESS

INISUB	EQU	02A4H		; ADDRESS OF INITIALIZATION SUBR IN WS 3.0

UNISUB	EQU	02A7H		;    "    "  DEINITIALIZATION SUBR IN 3.0

PRINIT	EQU	070DH		;    "	  "  PRINTER INITIALIZATION SUBR

PRFINI	EQU	0710H		;    "    "  "       DEINITIALIZATION SUBR

MORPAT	EQU	02E0H		;    "    "  EXTRA PATCHING AREA IN ROOT

LISEND	EQU	071DH		;    "    "  LST: PRINTER DRIVER IN WS

PBGMEM	EQU	035CH		;    "    "  POINTER TO FREE MEMORY

MEM	EQU	7900H		; WHERE THE ABOVE POINTER POINTS TO INITIALLY

;

ORG	INISUB

JMP	INITA		; CHANGE BDOS VECTOR TO GO TO OUR ROUTINE

; WHICH CHECKS FOR WS CHANGING DMA ADDRESS

; SO WE CAN RESET IT AFTER WE WRITE OUR FILE

; TO THE DISK.

;

ORG	UNISUB

JMP	DEINITA		; CHANGE BDOS VECTOR BACK TO WHAT IT WAS

; ORIGINALLY.

;

ORG	PRINIT

JMP	INITB		; DELETE THE FILE IF IT EXISTS, AND CREATE

; A NEW ONE.

;

ORG	PRFINI

JMP	DEINITB		; WRITE LAST SECTOR TO, AND CLOSE FILEOUT

;

ORG	LISEND

JMP	PFILE		; PRINT FILE TO DISK

;

ORG MORPAT

INITA:	LHLD	BDOS+1		; GET BDOS VECTOR

SHLD	CBDOS+1		; STORE IT FOR USE AFTER CHECKING FOR

; CHANGE DMA ADDRESS

LXI	H,TRANS		; POINT TO TRANSLATE ROUTINE

SHLD	BDOS+1		; MAKE 6 & 7 POINT THERE SO

; THAT WE CAN DO TRANSLATION

RET			; AND RETURN

;

INITB:	MVI	C,DELETEF	; DELETE "FILEOUT"

LXI	D,FILE		; IF IT EXISTS

CALL	BDOS		; NOTE, NO ERROR CHECKING

MVI	C,MAKEF		; CREATE THE FILE ON CURRENTLY LOGGED DRIVE

LXI	D,FILE

CALL	BDOS

INR	A		; IF NO DIRECTORY SPACE

JZ	EXIT		; RETURN TO OPERATING SYSTEM

RET			; ELSE DONE

;

DEINITA:LHLD	CBDOS+1		; POINT TO ORIGINAL JMP ADDRESS

SHLD	BDOS+1		; STORE IT BACK

RET

;

DEINITB:LXI	D,DEFBUF	; POINT TO BUFFER

MVI	C,SETDMA	; CHANGE DMA

CALL	CBDOS

LXI	D,FILE		; POINT TO FCB

MVI	C,WRITEF	; WRITE IT OUT

CALL	CBDOS

ORA	A		; ERROR IF NON-ZERO

JNZ	EXIT		; RETURN TO OPERATING SYSTEM

LXI	H,DEFBUF-1	; POINT TO 80H

SHLD	COUNT		; STORE NEW COUNT

LHLD	NEWDMA		; GET WS'S DMA ADDRESS

XCHG			; PUT IN DE

MVI	C,SETDMA	; TO SET DMA AND

CALL	CBDOS		; CHANGE IT BACK

MVI	C,CLOSEF	; CLOSE THE FILE

LXI	D,FILE

CALL	CBDOS

INR	A		; CHANGED DISKS ON US?

JZ	EXIT		; IF SO, REBOOT

RET			; ELSE DONE

;

TRANS:	PUSH	PSW		; SAVE ACCUMULATOR

MOV	A,C		; GET BDOS FUNCTION NUMBER

CPI	SETDMA		; IF SETTING DMA, WE WANT TO KNOW WHERE TO

JZ	STODMA		;

POP	PSW		; ELSE RESTORE ACC

CBDOS:	JMP	0000H		; AND JMP TO THE BDOS (ADDRESS PUT IN AT INITA)

;

STODMA:	XCHG			; GET NEW DMA IN HL

SHLD	NEWDMA		; STORE IT FOR FUTURE USE

XCHG			; RESTORE DE & HL TO ORIGINAL VALUES

POP	PSW		; RESTORE ACCUMULATOR

JMP	CBDOS		; AND JUMP TO BDOS

;

NEWDMA:	DW	0000H		; THIS IS WHERE WS'S DMA IS (PUT IN FROM

; STODMA ROUTINE ABOVE

;

ORG	PBGMEM

DW	LEN + MEM	; STORE NEW POINTER TO END OF MEMORY AT PBGMEM

;

ORG	MEM		; START WHERE PBGMEM ORIGINALLY POINTED TO

; BECAUSE THERE'S NOT ENOUGH SPACE TO DO

; ALL THIS IN MORPAT

;

; ACTUAL ROUTINE TO PRINT FILE TO DISK

;

PFILE:	LHLD	COUNT		; GET COUNT

INX	H		; INCREMENT IT

MOV	B,A		; SAVE CHARACTER BEING PRINTED

MOV	A,H		; GET HI ORDER COUNT

CPI	1		; GONE TO 100H?

JZ	WRITIT		; YES, TIME TO WRITE A SECTOR

SHLD	COUNT		; NO, STORE NEW COUNT

MOV	M,B		; STORE CHARACTER

RET			; AND RETURN

WRITIT:	PUSH	B		; SAVE LAST CHARACTER

LXI	D,DEFBUF	; POINT TO BUFFER

MVI	C,SETDMA	; CHANGE DMA

CALL	CBDOS

LXI	D,FILE		; POINT TO FCB

MVI	C,WRITEF	; WRITE IT OUT

CALL	CBDOS

ORA	A		; ERROR IF NON-ZERO

JNZ	EXIT		; RETURN TO OPERATING SYSTEM

LXI	H,DEFBUF-1	; POINT TO 80H

SHLD	COUNT		; STORE NEW COUNT

LHLD	NEWDMA		; GET WS'S DMA ADDRESS

XCHG			; PUT IN DE

MVI	C,SETDMA	; TO SET DMA AND

CALL	CBDOS		; CHANGE IT BACK

POP	B		; GET LAST CHARACTER

MOV	A,B		; INTO A

JMP	PFILE		; AND START AGAIN

;

EXIT:	CALL	DEINITA		; RESTORE BDOS VECTOR

JMP	0000		; AND REBOOT

;

COUNT:	DW	80H-1		; INITIALIZE TO BEGINNING - 1

FILE:	DB	0

DB	'FILEOUT ','   '; FCB FOR OUTPUT FILE

DB	0,0,0,0,0,0,0,0

DB	0,0,0,0,0,0,0,0

DB	0,0,0,0,0,0,0,0

DW	-1		; TO MARK END

LEN	EQU	$-PFILE

END

;

################################################