INSTALLATION AND USE OF MIC-XFER PROGRAM

------------ --- --- -- -------- -------

SYSTEM REQUIREMENTS:

IN ORDER TO USER MIC-XFER, THE FOLLOWING MINIMUM CONFIGURATION IS REQUIRED:

1. IBM-FORMAT CP/M SYSTEM CONFIGURED FOR AT LEAST 28K.

2. MICROPOLIS CP/M SYSTEM CONFIGURED FOR 17K.

3. THE SOURCE FILE 'MIC-XFER.ASM' (PROVIDED ON THIS DISK).

4. THE ABILITY FOR #1 AND #2 TO RESIDE IN THE SAME BUS AT

THE SAME TIME WITHOUT ANY HARDWARE CONFLICTS.

5. THE MICROPOLIS CONTROLLER ADDRESS (BOOTSTRAP ADDRESS) MUST BE

KNOWN.  (THIS MAY BE DETERMINED BY INSPECTING THE ADDRESS

JUMPERS ON THE MICROPOLIS CONTROLLER BOARD, AND INTERPRETING

THEM ACCORDING TO THE MICROPOLIS MANUAL).

6. THE ADDRESS OF THE IBM-FORMAT CP/M WARM START VECTOR MUST BE

KNOWN.  (THIS MAY BE DETERMINED BY EXAMINING LOCATIONS

1 AND 2 IN MEMORY WHILE THE IBM-FORMAT SYSTEM IS RUNNING).

THE FIRST STEP IN GENERATING MIC-XFER FOR YOUR SYSTEM IS TO GET A COPY OF

YOUR 17K MICROPOLIS SYSTEM IMAGE ONTO AN IBM-FORMAT DISK.  THIS MAY BE DONE

AS FOLLOWS:

1. BOOT THE IBM-FORMAT SYSTEM.

2. TRANSFER CONTROL TO THE MICROPOLIS SYSTEM BY TYPING:

A>DDT

DDT VERS 1.4

-G<MICROPOLIS BOOT ADDRESS>

THE MICROPOLIS SYSTEM WILL BOOT UP AND SIGN ON.

3. CREATE A SYSTEM IMAGE FILE BY TYPING:

A>SYSGEN

SYSGEN VERS 1.4

SOURCE DRIVE NAME (OR RETURN TO SKIP) A

FUNCTION COMPLETE

DESTINATION DRIVE NAME (OR RETURN TO REBOOT) <RETURN>

A>SAVE 36 MIC17.COM

4. TRANSFER MIC17.COM TO THE IBM-FORMAT SYSTEM BY TYPING (FROM THE

MICROPOLIS SYSTEM):

A>DDT MIC17.COM

DDT VERS 1.4

NEXT PC

2500 0100

-G<IBM-FORMAT WARM START ADDRESS>

THE IBM-FORMAT SYSTEM SHOULD WARM-BOOT AND PROMPT.  TYPE:

A>SAVE 36 MIC17.COM

NOW THAT YOU HAVE A COPY OF YOUR MICROPOLIS CP/M ON AN IBM-FORMAT DISK, YOU

MUST CUSTOMIZE MIC-XFER.ASM FOR YOUR MICROPOLIS CONTROLLER, AND THEN ASSEMBLE

IT AND COMBINE IT WITH MIC17.COM.  THIS IS DONE AS FOLLOWS:

1. DETERMINE THE CORRECT VALUE FOR THE SYMBOL 'MCTL' IN MIC-XFER.

THIS MAY BE CALCULATED BY ADDING 200H TO THE MICROPOLIS BOOT

ADDRESS.  FOR EXAMPLE, IF YOUR MICROPOLIS BOOT ADDRESS IS F800,

THEN THE CORRECT VALUE OF 'MCTL' IS FA00.

2. EDIT THE CORRECT VALUE OF 'MCTL' FOR YOUR SYSTEM INTO MIC-XFER.ASM.

THE 'EQU' FOR 'MCTL' MAY BE FOUND ON LINE 6.

3. WHILE YOU ARE IN THE EDITOR, YOU MAY WANT TO COMMENT OUT THE 'TITLE'

STATEMENT IN LINE 1.  IF YOU ASSEMBLE MIC-XFER USING 'ASM', THIS

LINE WILL PRODUCE AN ERROR, BUT THE CORRECT CODE WILL STILL BE

GENERATED.  IF YOU ASSEMBLE MIC-XFER USING 'MAC', THE 'TITLE'

STATEMENT WILL BE ACCEPTED PROPERLY, AND NEED NOT BE COMMENTED OUT.

4. USING EITHER 'ASM' OR 'MAC', ASSEMBLE MIC-XFER TO PRODUCE THE

OBJECT FILE 'MIC-XFER.HEX'.

5. MERGE MIC-XFER.HEX WITH MIC17.COM BY TYPING:

A>DDT MIC17.COM

DDT VERS 1.4

NEXT PC

2500 0100

-IMIC-XFER.HEX

-R

-G0

A>SAVE 36 MIC-XFER.COM

THIS COMPLETES INSTALLATION OF MIC-XFER IN YOUR SYSTEM.

USING MIC-XFER:

MIC-XFER WILL TRANSFER ONE OR MORE FILES BETWEEN AN IBM-FORMAT CP/M SYSTEM AND

A MICROPOLIS CP/M SYSTEM.  THE SOURCE DRIVE, DESTINATION DRIVE, AND DIRECTION

OF TRANSFER MAY BE SPECIFIED IN THE COMMAND LINE.  THE CP/M WILDCARDS '?' AND

'*' ARE FULLY IMPLEMENTED, MAKING MULTIPLE FILES OR EVEN WHOLE-DISK TRANSFERS

VERY EASY.  MIC-XFER IS EXECUTED BY TYPING:

A>MIC-XFER S:FFFFFFFF.TTT D:X

WHERE:

S	  IS THE SOURCE DRIVE

FFFFFFFF  IS THE AMBIGUOUS OR NON-AMBIGUOUS FILENAME

TTT	  IS THE AMBIGUOUS OR NON-AMBIGUOUS FILETYPE

D	  IS THE DESTINATION DRIVE

X	  IS THE DESTINATION SYSTEM - I=IBM-FORMAT

M=MICROPOLIS

FOR EXAMPLE, TO TRANSFER ALL '.COM' FILES ON IBM-FORMAT DRIVE B TO MICROPOLIS

DRIVE A, YOU WOULD TYPE:

A>MIC-XFER B:*.COM A:M

WHILE EXECUTING, MIC-XFER WILL REPORT EACH FILENAME AS IT IS TRANSFERRED,

AS WELL AS ANY ERRORS WHICH MAY OCCUR.  AN ERROR ON ONE FILE OF A MULTI-

FILE TRANSFER WILL NOT ABORT THE TRANSFER OF THE REMAINING FILES.  EXECUTION

MAY BE INTERRUPTED BY STRIKING CONTROL-C, WHICH WILL BE RECOGNIZED AT THE

COMPLETION OF THE CURRENT FILE TRANSFER.