A68K - 68000 Cross Assembler Notes

11/09/82

Command line to invoke the assembler:

A68K <filename>                   ; short form

A68K <object>,<listing>=<source>  ; full form

----------------- Equivalence ----------------

XYZ                 XYZ.HEX,XYZ.PRN=XYZ.ASM

,CON:=XYZ           NUL:,CON:=XYZ.ASM

XYZ,=XYZ            XYZ.HEX,NUL:=XYZ.ASM

XYZ.OBJ=XYZ.SRC     XYZ.OBJ,XYZ.PRN=XYZ.SRC

,CON:=CON:          NUL:,CON:=CON:

Object file may be in Motorola S-record (default)

or Intel HEX format.

<symbol>  SET   <expression>            ; temp val

<symbol>  EQU   <expression>            ; perm val

<symbol>  REG   <register expression>   ; perm val

Note that a size attribute may be tacked onto

SET or EQU to govern the way string constants

appearing in the expression are handled.  The

default size is long.  String  constants  are

left justified  according  to  size  and zero

filled.

<register expression>          for use in MOVEM

<Rn>    <next reg>

<next reg>

- <Rn>   <next reg>

/        <register expression>

<Rn>

A <rnum>

D <rnum>

Labels are symbols which receive the current value of

the program counter.

For labels beginning in column 1, colon is optional.

For labels preceded by blanks,    colon is required.

OPT   <option>,<option>,.....

<option>

BRL     ; long  branch forward ref. - default

BRS     ; short branch forward ref.

FRL     ; long  absolute fwd. ref.  - default

FRS     ; short absolute fwd. ref.

OBJ-S   ; hex file Motorola style   - default

OBJ-H   ; hex file Intel style (16 bit address limit)

NOP1L   ; no pass 1 listing         - default

P1L     ;    pass 1 listing

* <comment>

.S & .L  may be appended to BRA, BSR, Bcc, JMP and JSR

to override automatic size determination.

They  may  also  be   appended  to  absolute  address

operands in other instructions.  But, it is necessary

to isolate  the  period  from a  symbol  by enclosing

the symbol in parentheses.  This  is  due to the fact

that period is a valid character in symbols.

<symbol>

Symbols begin with a letter or period.  Additional

characters may be letters, digits, periods, dollar

signs and underscores.

* represents the program counter

Symbols may have up to 31 significant characters.

The symbol table is currently of fixed size:

500 Symbols max

2000 Characters max

Constants begin with:

char    - '

hex     - $

octal   - @

binary  - %

decimal - 0 thru 9

Strings:

Delimited by '

'' represents '

Expressions and symbols are classified as constants

or addresses.

Operators for expressions (in precedence group order):

+ -

* /        only implemented for 16 bits now

\          mod - only 16 bits for now

&          AND

! |        both allowed for OR

^          exclusive OR

<<         shift left

>>         shift right

-          unary minus

~          unary NOT

Except for + and -,  only  expressions  which evaluate

constants are allowed as  operands to  operators.  For

+ and -, (a + a) and (c - a) produce errors.  Embedded

spaces are not allowed in expressions (spaces serve as

field delimiters).

Automatic conversions:

ADD     #e   ,<ea>      ADDQ or ADDI when possible

<ea> ,An        ADDA

SUB     #e   ,<ea>      SUBQ or SUBI when possible

<ea> ,An        ADDA

CMP     #e   ,<ea>      CMPI when possible

(An)+,(An)+     CMPM

<ea> ,An        CMPA

AND     #e   ,<ea>      ANDI when possible

OR      #e   ,<ea>      ORI  when possible

EOR     #e   ,<ea>      EORI when possible

Addressing Mode Syntax Interpretation:

c  expression must evaluate to constant

a  expression must evaluate to address

b  expression may  evaluate to either

Syntax used            Interpreted as    Mode,reg

# e                     #c              7,4

D n                     Dn              0

A n                     An              1

- ( A n )               -(An)           4

( A n ) +               (An)+           3

( A n )                 (An)            2

( A n , R i )

( A n , R i . W )       0(An,Ri.s)      6

( A n , R i . L )

e ( A n , R i )

e ( A n , R i . W )     c(An,Ri.s)      6

e ( A n , R i . L )

e ( A n )               c(An)           5

e ( D i )

e ( R i . W )           a(PC,Ri.s)      7,3

e ( R i . L )

e ( PC , R i )

e ( PC , R i . W )      a(PC,Ri.s)      7,3

e ( PC , R i . L )

e ( PC )                a(PC)           7,2

e . L                   b.L             7,1          **

e . S                   b.S             7,0          **

e                       b               7,0 or 7,1   **

****   SP may be used for A7 anywhere An appears, but not

where Ri appears for index specification.

**     Explicit  range   specification   via   (.S or .L)

overrides  the  automatic range  selection  for  absolute

addressing.  For auto, if the expression contains forward

references, the  range is  determined by  the "FR" option

currently in  effect  (FRL default).   If  the expression

contains  no  forward references, the range is made short

if the expression lies between -32768 and +32767.